S. Rupika

192512398

ECA5403-ELECTROMAGNETIC FILEDS AND WAVES

C02-7

Exp:7- Determination and plotting of Capacitance of isolated sphere and co-axial cable with different

dimensions and dielectric constant.

Aim:

To write program for capacitance of isolated sphere and co-axial cable with different radius and dielectric

constant using MATLAB.

Software required:

•MATLAB (R2018 or later)

Test case (i) Determination of capacitance of isolated sphere for given radius

Program:

clc;

clear;

epsilon0 = 8.854e-12;

R = input('Enter radius of sphere (m):');

C = 4*pi*epsilon0*R;

disp('Capacitance of isolated sphere is');

disp(C);

Test case (ii) Determination and plotting of capacitance of isolated sphere with varying

radius

clc;

epsilon0 = 8.854e-12;

R = linspace(0.1,2,100);

C = 4*pi*epsilon0*R;

plot(R,C,'LineWidth',2);

xlabel('Radius (m)');

ylabel('Capacitance (F)');

title('Capacitance of isolated sph

OUTPUT:

Test case (iii) Determination of capacitance per unit length of co-axial

cable clc;

epsilon0 = 8.854e-12;

a = input('Enter inner radius (m):');

b = input('Enter outer radius (m):');

er = input('Enter dielectric constant:');

epsilon = epsilon0*er;

C = (2*pi*epsilon)/log(b/a);

disp('Capacitance per unit length is');

disp(C);

Test case (iv) Determination and plotting of capacitance variation with dielectric constant

in co-axial cable

clc;

epsilon0 = 8.854e-12;

a = 0.01;

b = 0.05;

er = linspace(1,5,100);

epsilon = epsilon0*er;

C = (2*pi*epsilon)./log(b/a);

plot(er,C,'LineWidth',2);

xlabel('Dielectric constant');

ylabel('Capacitance (F/m)');

title('Capacitance vs dielectric constant (coaxial cable)');

OUTPUT:

RESULT:

Thus the program was verified for capacitance of isolated sphere and co-axial cable using MATLAB was successfully